13:40-15.16 STANDARDS OF PRACTICE

a) All home inspectors shall comply with the standards of practice contained in this section

when conducting home inspections. The scope of home inspection services performed in

compliance with the requirements set forth in this section shall provide the client with

objective information regarding the condition of the systems and components of the

home as determined at the time of the home inspection.

b) Nothing in this section shall be construed to require a home inspector to:

1) Enter any area or perform any procedure which is, in the opinion of the home

inspector, unsafe and likely to be dangerous to the inspector or other persons;

2) Enter any area or perform any procedure which will, in the opinion of the home

inspector, likely damage the property or its systems or components;

3) Enter any area which does not have at least 24 inches of unobstructed vertical

clearance and at least 30 inches of unobstructed horizontal clearance;

4) Identify concealed conditions and latent defects;

5) Determine life expectancy of any system or component;

6) Determine the cause of any condition or deficiency;

7) Determine future conditions that may occur including the failure of systems and

components including consequential damage;

Page 19 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

8) Determine the operating costs of systems or components;

9) Determine the suitability of the property for any specialized use;

10) Determine compliance with codes, regulations and/ or ordinances;

11) Determine market value of the property or its marketability;

12) Determine advisability of purchase of the property;

13) Determine the presence of any potentially hazardous plants, animals or diseases or

the presence of any suspected hazardous substances or adverse conditions such as

mold, fungus, toxins, carcinogens, noise, and contaminants in soil, water, and air;

14) Determine the effectiveness of any system installed or method utilized to control or

remove suspected hazardous substances;

15) Operate any system or component which is shut down or otherwise inoperable;

16) Operate any system or component which does not respond to normal operating

controls;

17) Operate shut-off valves;

18) Determine whether water supply and waste disposal systems are public or private;

19) Insert any tool, probe or testing device inside electrical panels;

20) Dismantle any electrical device or control other than to remove the covers of main

and sub panels;

21) Walk on unfloored sections of attics; and

22) Light pilot flames or ignite or extinguish fires.

Page 20 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

c) Home inspectors shall:

1) Inspect the following systems and components in residential buildings and other

related residential housing components:

i)

ii)

Structural components as required by (e) below;

Exterior components as required by (f) below;

iii) Roofing system components as required by (g) below;

iv) Plumbing system components as required by (h) below;

v) Electrical system components as required by (i) below;

vi) Heating system components as required by (j) below;

vii) Cooling system components as required by (k) below;

viii) Interior components as required by (l) below;

ix) Insulation components and ventilation system as required by (m) below; and

x) Fireplaces and solid fuel burning appliances as required by (n) below;

2) Prepare a home inspection report which shall:

i)

ii)

Disclose those systems and components as set forth in (c)1 above which were

present at the time of inspection;

Disclose systems and components as set forth in (c)1 above which were present

at the time of the home inspection but were not inspected, and the reason(s) they

were not inspected;

iii) Describe the systems and components specified in these standards of practice;

iv) State material defects found in systems or components;

Page 21 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

v) State the significance of findings where any material defects in the systems and

components of (c)1 above were found; and

vi) Provide recommendations where material defects were found to repair, replace or

monitor a system or component or to obtain examination and analysis by a

qualified professional, tradesman, or service technician without determining the

methods, materials or cost of corrections; and

3) Retain copies of all home inspection reports prepared pursuant to (c)2 above, for a

period of five years upon completion of the report;

d) Subsection (c) above is not intended to limit home inspectors from:

1) Inspecting or reporting observations and conditions observed in systems and

components in addition to those required in (c)1 above and inspecting systems and

components other than those mandated for inspection in (c)1 above as long as the

inspection and reporting is based on the licensee's professional opinion, prior work

experience, education and training, unless these standards of practice prohibit the

home inspector from inspecting such system or component;

2) Contracting with the client to provide, for an additional fee additional inspection

services provided the home inspector is educated, trained, certified, registered or

licensed, pursuant to the provisions of N.J.A.C. 13:40-15.22 and other applicable

statutes and rules; and

3) Excluding systems and components from the inspection if requested in writing by the

client.

e) When conducting the inspection of the structural components, the home inspector shall:

1) Inspect:

i) Foundation;

ii) Floors;

iii) Walls;

iv) Ceilings; and

Page 22 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

v) Roof;

2) Describe:

i)

ii)

Foundation construction type and material;

Floor construction type and material;

iii) Wall construction type and material;

iv) Ceiling construction type and material; and

v) Roof construction type and material;

3) Probe structural components where deterioration is suspected unless such probing

would damage any finished surface; and

4) Describe in the home inspection report the methods used to inspect under-floor crawl

spaces and attics.

f)

When conducting the inspection of the exterior components, a home inspector shall:

1) Inspect:

i)

ii)

Exterior surfaces, excluding shutters, and screening, awnings, and other similar

seasonal accessories;

Exterior doors excluding storm doors or safety glazing;

iii) Windows excluding storm windows and safety glazing;

iv) Attached or adjacent decks, balconies, stoops, steps, porches, and their railings;

v) Vegetation, grading, drainage, and retaining walls with respect to their immediate

detrimental effect on the condition of the residential building, excluding fences,

geological and/or soil conditions, sea walls, break-walls, bulkheads and docks, or

erosion control and earth stabilization;

Page 23 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

vi) Attached or adjacent walkways, patios, and driveways; and

vii) Garage doors including automatic door openers and entrapment protection

mechanisms, excluding remote control devices; and

2) Describe exterior wall surface type and material.

g) When inspecting the roof of a residential building, the home inspector shall:

1) Inspect:

i)

ii)

Roofing surface, excluding antennae and other installed accessories such as

solar heating systems, lightning arresters, and satellite dishes;

Roof drainage systems;

iii) Flashing;

iv) Skylights; and

v) Exterior of chimneys;

2) Describe:

i) Roof surface;

ii)

Roof drainage systems;

iii) Flashing;

iv) Skylights; and

v) Chimneys;

3) Employ reasonable, practicable and safe methods to inspect the roof such as:

i)

Walking on the roof;

Page 24 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

ii)

Observation from a ladder at roof level; or

iii) Visual examination with binoculars from ground level; and

4) Describe the methods used to inspect the roof.

h) When inspecting the plumbing system, a home inspector shall:

1) Inspect:

i)

ii)

Interior water supply and distribution systems including functional water flow and

functional drainage, excluding wells, well pumps, well water sampling or water

storage related equipment, determination of water supply quantity or quality and

water conditioning systems and lawn irrigation systems;

All interior fixtures and faucets, excluding shut off valves, wells, well pumps, well

water sampling and water storage related equipment;

iii) Drain, waste and vent systems;

iv) Domestic water heating systems, without operating safety valves or automatic

safety controls, and excluding solar water heating systems;

v) Combustion vent systems excluding interiors of flues and chimneys;

vi) Fuel distribution systems; and

vii) Drainage sumps, sump pumps and related piping; and

2) Describe:

i)

ii)

Predominant interior water supply and distribution piping materials;

Predominant drain, waste and vent piping materials; and

iii) Water heating equipment including energy sources.

i)

When inspecting the electrical system, a home inspector shall:

Page 25 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

1) Inspect:

i)

ii)

Service entrance system;

Main disconnects, main panel and sub panels, including interior components of

main panel and sub panels;

iii) Service grounding;

iv) Wiring, without measuring amperage, voltage or impedance, excluding any wiring

not a part of the primary electrical power distribution system, such as central

vacuum systems, remote control devices, telephone or cable system wiring,

intercom systems, security systems and low voltage wiring systems;

v) Over-current protection devices and the compatibility of their ampacity with that of

the connected wiring;

vi) At least one of each interior installed lighting fixture, switch, and receptacle per

room and at least one exterior installed lighting fixture, switch, and receptacle per

side of house; and

vii) Ground fault circuit interrupters; and

2) Describe:

i)

ii)

Amperage and voltage rating of the service;

Location of main disconnect, main panels, and sub-panels;

iii) Type of over-current protection devices;

iv) Predominant type of wiring;

v) Presence of knob and tube branch circuit wiring; and

vi) Presence of solid conductor aluminum branch circuit wiring.

j)

When inspecting the heating system, a home inspector shall:

Page 26 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

1) Inspect:

i)

ii)

Installed heating equipment and energy sources, without determining heat supply

adequacy or distribution balance, and without operating automatic safety controls

or operating heat pumps when weather conditions or other circumstances may

cause damage to the pumps, and excluding humidifiers, electronic air filters and

solar heating systems;

Combustion vent systems and chimneys, excluding interiors of flues or chimneys;

iii) Fuel storage tanks, excluding propane and underground storage tanks; and

iv) Visible and accessible portions of the heat exchanger; and

2) Describe:

i)

Heating equipment and distribution type; and

ii) Energy sources.

k) When inspecting the cooling system, a home inspector shall:

1) Inspect:

i)

ii)

Central cooling system, excluding electronic air filters and excluding

determination of cooling supply adequacy or distribution balance and without

operating central cooling equipment when weather conditions or other

circumstances may cause damage to the cooling equipment;

Permanently installed hard-wired, through-wall individual cooling systems; and

iii) Energy sources; and

2) Describe:

i)

Cooling equipment and distribution type; and

ii) Energy sources.

Page 27 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

l)

When inspecting the interior of a residential building, a home inspector shall:

1) Inspect:

i)

ii)

Walls, ceilings, and floors excluding paint, wallpaper and other finish treatments,

carpeting and other non-permanent floor coverings;

Steps, stairways, and railings;

iii) Installed kitchen wall cabinets to determine if secure;

iv) At least one interior passage door and operate one window per room excluding

window treatments; and

v) Household appliances limited to:

(1) The kitchen range and oven to determine operation of burners or heating

elements excluding microwave ovens and the operation of self-cleaning

cycles and appliance timers and thermostats;

(2) Dishwasher to determine water supply and drainage; and

(3) Garbage disposer.

m) When inspecting the insulation components and ventilation system of a residential

building, the home inspector shall:

1) Inspect:

i)

ii)

Insulation in unfinished spaces without disturbing insulation;

Ventilation of attics and crawlspaces; and

iii) Mechanical ventilation systems; and

2) Describe:

i)

Insulation in unfinished spaces adjacent to heated areas; and

Page 28 of 38

Chapter 40 Subchapter 15

Last Revision Date: 7/15/13

[Home Inspection Advisory Committee]

LAW AND PUBLIC SAFETY

ii)

Evidence of inadequate attic and crawlspace ventilation.

n) When inspecting fireplaces and solid fuel burning appliances, a home inspector shall:

1) Inspect:

i)

ii)

Fireplaces and solid fuel burning appliances, without testing draft characteristics,

excluding fire screens and doors, seals and gaskets, automatic fuel feed devices,

mantles and non-structural fireplace surrounds, combustion make-up air devices,

or gravity fed and fan assisted heat distribution systems; and

Chimneys and combustion vents excluding interiors of flues and chimneys; and

2) Describe:

i)

Type of fireplaces and/or solid fuel burning appliances;

ii) Energy source; and

iii) Visible evidence of improper draft characteristics.